UART

UART stands for Universal Asynchronous Receiver-Transmitter.

It is a hardware communication protocol used for asynchronous serial data transfer between two devices.

“Asynchronous” means it does not use a shared clock signal; instead, both devices must agree on timing (baud rate) beforehand

Serial communication sends data one bit at a time over a single line, unlike parallel communication, which uses many wires simultaneously.

UART simplifies data exchange using just two main lines:

- TX (Transmit) – sends data out.

- RX (Receive) – receives data in

Data Framing

Each character transmitted is packaged in a frame with:

- Start bit – signals beginning of transmission (logic LOW).

- Data bits – typically 5 to 9 bits representing the actual data.

- Optional parity bit – used for basic error checking.

- Stop bit(s) – one or two bits marking the end of a frame (logic HIGH).

** Idle State When no data is sent, the UART line stays idle (logic HIGH).**

Baud Rate & Timing

- Baud rate = number of bits transmitted per second (e.g., 9600, 115200).

- Since the protocol is asynchronous, both devices must set the same baud rate for proper communication

Transmission Types

UART supports simple transmission modes:

- Simplex – one-way data flow.

- Half-duplex – both can send/receive, but not at the same time.

- Full-duplex – simultaneous send & receive.

Key features

- Simple, low-pin communication (only RX/TX + ground needed).

- Widely used in microcontrollers, FPGAs, sensors, GPS, serial debug ports.

- Does not require clock line between devices.

- Error checking via optional parity bit

Bit Rate (Baud Rate)

Number of bits transmitted per second.

Bit Rate (bps)=1Bit Time
Example: If baud rate = 9600

Bit Rate=9600 bits/second

Bit Time

Time required to transmit one bit

Bit Time=1Bit Rate
Example 1: 9600 baud

Bit Time=19600
=0.00010416 sec
=104.16 microseconds

Total Transmission Time

Now in UART, one character frame usually contains:

- 1 Start bit

- 8 Data bits

- 1 Stop bit

- (Optional parity bit)

So total bits per frame:

Without parity:

1+8+1=10 bits

With parity:

1+8+1+1=11 bits

📌 Transmission Time Formula

Total Time=Number of Bits×Bit Time

OR

Total Time=Number of BitsBit Rate

Example Calculation (9600 baud, 8N1 format)

8N1 = 8 data bits, No parity, 1 stop bit
Total bits = 10

Total Time=109600
=0.0010416 sec
=1.0416 milliseconds per character

If Sending Multiple Bytes

Suppose sending 100 bytes at 9600 baud:

Each byte = 10 bits
Total bits = 100 × 10 = 1000 bits

Total Time=10009600
=0.10416 sec
=104.16 ms
Quick Reference Table

| Baud Rate | Bit Time |
| --- | --- |
| 9600 | 104.16 µs |
| 19200 | 52.08 µs |
| 115200 | 8.68 µs |

Baud rate Generator design

  Clock = 50 MHz

  Baud rate = 9600 bps

TX

Divisor=50,000,0009600=5208.33

Counter max value = 5207

RX

Divisor=50,000,0009600*16=325

Counter max value = 324

| Mode | Count From | Count To | Total Cycles | Counter Bits |
| --- | --- | --- | --- | --- |
| Normal UART | 0 | 5207 | 5208 | 13 bits |
| 16x Oversampling | 0 | 324 | 325 | 9 bits |

Transmitter design

Objective

To design a synchronous UART transmitter module that serially transmits 8-bit parallel data using:

- 1 Start bit

- 8 Data bits

- 1 Stop bit

The design operates using a baud-rate enable signal (enb) and indicates transmission status using tx_busy.

Functional Overview

The transmitter converts 8-bit parallel data into a serial data stream following the standard UART frame format:

| Start | D0 | D1 | D2 | D3 | D4 | D5 | D6 | D7 | Stop |

|   0   |                       8-bit data                          |  1  |

Transmission begins when:

- wr_enb = 1 (data write request)

- Module is in idle_state

Each bit is transmitted only when enb = 1 (baud clock enable pulse).

Port Description

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | Input | 1 | System clock |
| reset | Input | 1 | Active-low asynchronous reset |
| wr_enb | Input | 1 | Write enable to start transmission |
| enb | Input | 1 | Baud-rate enable signal |
| data_in | Input | 8 | Parallel data to transmit |
| tx | Output | 1 | Serial transmit line |
| tx_busy | Output | 1 | Indicates transmitter is busy |

Internal Signals

| Signal | Width | Description |
| --- | --- | --- |
| data | 8-bit | Stores input data |
| index | 3-bit | Bit counter (0–7) |
| state | 2-bit | FSM state register |

Finite State Machine (FSM)

| State | Encoding | Description |
| --- | --- | --- |
| IDLE | 00 | Waits for write enable |
| START | 01 | Sends start bit (0) |
| DATA | 10 | Sends 8 data bits (LSB first) |
| STOP | 11 | Sends stop bit (1) |

IDLE  --wr_enb-->  START

START --enb--> DATA

DATA  --8 bits--> STOP

STOP  --enb--> IDLE

Receiver design

Objective

To design a UART receiver module that converts serial input data (rx) into 8-bit parallel output using:

- 1 Start bit

- 8 Data bits

- 1 Stop bit

The design uses 16x oversampling for reliable data recovery and generates a rdy signal when valid data is received.

| Start | D0 | D1 | D2 | D3 | D4 | D5 | D6 | D7 | Stop |

|   0   |               8-bit Data (LSB First)             |  1  |

Port Description

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | Input | 1 | System clock |
| reset | Input | 1 | Active-low asynchronous reset |
| rx | Input | 1 | Serial input line |
| rdy_clr | Input | 1 | Clears ready signal |
| clk_en | Input | 1 | Baud-rate sampling enable (16x) |
| rdy | Output | 1 | High when valid data received |
| data_out | Output | 8 | Received parallel data |

Internal Registers

| Signal | Width | Description |
| --- | --- | --- |
| state | 2-bit | FSM state register |
| sample | 4-bit | Oversampling counter (0–15) |
| index | 3-bit | Bit counter (0–7) |
| temp_register | 8-bit | Temporary data storage |

Finite State Machine (FSM)

| State | Encoding | Description |
| --- | --- | --- |
| START_STATE | 00 | Detect start bit |
| DATA_OUT_STATE | 01 | Receive 8 data bits |
| STOP_STATE | 10 | Validate stop bit |

State Operation

Start State

- Wait for rx = 0 (start bit detection)

- Increment sample counter

- When sample == 7 (middle of start bit):

  - Reset counters

  - Move to DATA state

Data State

- sample counts from 0 to 15 (16x oversampling)

- When sample == 15:

  - Sample rx

  - Store into temp_register[index]

  - Increment index

- After 8 bits → move to STOP state

Stop State

- Wait until sample == 15

- Transfer temp_register → data_out

- Set rdy = 1

- Return to START state

- rdy_clr clears ready flag

Verification

Testbench

├── Generator

├── Driver

├── Monitor

├── Scoreboard

└── DUT

Transaction/: A transaction is a data packet.

Transaction = One packet of information

It represents:

- Input stimulus (like data, control signals)

- Or output results

Generator:

The generator creates stimulus.

Its job:

- Create transactions

- Randomize values

- Send them to the driver

It does NOT know about signals.
It only works with data packets.

Driver

The driver converts transactions into actual signal-level activity.

Its job:

- Receive transaction from generator

- Apply signals to DUT

- Handle clock timing

- Respect protocol rules

DUT (Design Under Test)

The DUT processes input signals and produces output signals.

Monitor

The monitor observes DUT signals.

Its job:

- Watch output signals

- Convert signals back into transaction format

- Send them to scoreboard

Scoreboard

The scoreboard checks correctness.

Its job:

- Compare expected data vs actual data

- Report PASS or FAIL

- Detect errors

It ensures the DUT behaves correctly.

Mailbox

A mailbox is a communication mechanism used to pass data between processes.